LabVIEW Real-Time for ETS target

The experimental setup uses the following hardware:

- ECP Model 210, or Model 205 Rectilinear Plant and amplifier box

- NI PXI-1042 chassis

- NI PXI-8186 embedded controller

- NI PXI-7831R reconfigurable I/O module

- Dell Optiplex Pentium 4 workstation with Windows XP

- Custom RIO to ECP cable (connects to MIO connection on 7831r)

In addition, the following software is necessary:

- LabVIEW 7.1 Professional Development System

- LabVIEW Simulation Module

- LabVIEW 7.1 Real-Time Module for ETS

- LabVIEW 7.1 FPGA Module

- NI-RIO 1.2

- LabVIEW 7.1 Control Design Toolkit

- LabVIEW 7.1 Simulation Module

- Install Software in the following order:

  - LabVIEW & Drivers

    - LabVIEW 7.1 (CD in Bundle)

    - Device Driver CD 1 >> Device Driver CD 2 (CDs in Bundle)

  - LabVIEW Toolsets

    - Simulation Module (CD in Bundle)

    - System ID Toolset (Toolkit Software CD2, LabVIEW folder)

    - Control Design Toolkit (Toolkit Software CD2, LabVIEW folder)

  - LabVIEW Real-Time Module

    - LabVIEW Real-Time Module (ETS) for Windows (CD in Bundle)

    - Device Driver CD 1 >> Device Driver CD 2 (CDs in Bundle, same as before)

  - LabVIEW FPGA Module

    - LabVIEW 7.1 FPGA Module (CD in Bundle)

  - R-Series RIO Driver (i.e. NI-RIO 1.3) (Download from NI.com)

    - http://search.ni.com/nisearch/nisearchservlet?nistype=default&filter=+%2BProductLine%3A2712+%2Bcontentformat%3Asoftwarelib&ddown=2

LabVIEW Real-Time for RTX Target

The experimental setup uses the following hardware:

- ECP Model 210 Rectilinear Plant and amplifier box

- Dell Optiplex Pentium 4 workstation with Windows XP

- Custom RIO to ECP cable (connects to MIO connection on 7831r)

In addition, the following software is necessary:

- Download and install RTX from Ardance

- LabVIEW 7.1 Professional Development System

- LabVIEW Simulation Module

- LabVIEW 7.1 Real-Time Module for RTX

- LabVIEW 7.1 FPGA Module

- NI-RIO 1.2

- LabVIEW 7.1 Control Design Toolkit

- LabVIEW 7.1 Simulation Module

- Install Software in the following order:

  - See “Installing and Adding a PCI device to  RTX 5.5 Setup Procedure.doc” or “Installing and Adding a PCI device to  RTX 6.1 Setup Procedure.doc” in the Real-Time RTX Troubleshooting folder